New Fuel Receipt Survey:

Question 1:  What role, if any, does Quality Assurance or Quality Control play in the fuel receipt process?  Specifically, do they perform any inspections, review paperwork, etc?

| Clinton | Clinton Reactor Engineering is responsible for review of the fuel receipt paperwork including the PQC verification. Quality Assurance is no longer in the business of supporting fuel receipt activities under Exelon Management. |
| --- | --- |
| Cooper | QA reviews qualifications (experience/education background) and certifies fuel inspectors upon completion of the vendor training. They do not perform any inspections, but move sheets and inspections documents are considered quality documents. |
| River Bend | QA qualifies our receipt inspectors. |
| Grand Gulf | None |
| Waterford | None. QA usually does spot checks associated with new fuel receipt/procedure completion, but is not directly required for new fuel receipt. |
| Fitzpatrick | Just in fabrication process. |
| Byron | Neither QA nor QC play a role in fuel receipt activities or paperwork. |
| Wolf Creek | QA provides no role except to audit procedure changes if any. QC performs the official inspection of the fuel assemblies. |
| Oconee | At ONS, the RxE are qualified as receipt inspectors for new fuel. The QA/QC organization trains and qualifies the RxEs and maintains the training records and receipt paperwork after we're done receiving the fuel. They double check our paperwork after the fact. |

Question 2: What is the purpose the new fuel inspection activity?  Is the new fuel receipt inspection considered an inspection for "shipping or handling damage" or it is a traditional receipt inspection?

| Clinton | The purpose of fuel receipt inspection is to inspect for shipping and handling damage and as well obvious manufacturing errors (i.e. poor engagement of expansion springs to tie rods, bent spacer tabs, etc.). |
| --- | --- |
| Cooper | Fuel inspection is not treated the same as a standard receipt inspection and is covered by Fuels and Reactor Engineering procedures. The inspections look for shipping and handling damage, as well as FME and visible manufacturing defects (scratches greater than 2 mils, issues with plenum springs, etc.). |
| River Bend | Shipping and handling. We do examine critical areas. See copy of attached procedure. |
| Grand Gulf | Shipping inspection |
| Waterford | This inspection is for shipping and handling damage, as well as to look for debris, grid issues, etc. |
| Fitzpatrick | Whatever GNF recommends. |
| Byron | Nuclear fuel and associated fuel components are inspected upon receipt at each nuclear site to accomplish the following; A2.1 Confirmation the fuel design and bundle serial numbers are as ordered A2.2 Confirmation that there has been no shipping or handling damage A2.3 Confirmation of certain design or fabrication details are as specified by Nuclear Fuels (NF) A2.4 Confirmation that there is no foreign material present and that overall cleanliness is acceptable A2.5 Cleanup of any minor foreign material or surface contamination A2.6 Correction of minor issues such as lock tab engagement on tie rod nuts or BWR bundle twist. |
| Wolf Creek | Inspection is to verify fuel assemblies have sustained no damage during loading or unloading into the shipping containers. It also serves to inspect for any abnormalities that may have been missed at the fabrication process. |
| Oconee | Both inspection for shipping & handling and traditional receipt inspection to the extent possible. |

Question 3: Who does "inspection" (i.e. Reactor Engineering, QA/QC, Maintenance, shared resources, etc.)?

| Clinton | The actual inspection of the fuel is done entirely by Mechanical Maintenance under the management of Exelon. |
| --- | --- |
| Cooper | Individuals at Cooper who meet QA requirements (see #1) and can be qualified to inspect new fuel. |
| River Bend | Maintenance. |
| Grand Gulf | Maintenance - Coordinated by a Fuel Services Supervisor (new group for DFS, etc.) |
| Waterford | Reactor Engineering and shared resources perform the inspections. |
| Fitzpatrick | Maintenance |
| Byron | The Reactor Services Department. Reactor Services performs the receipt inspection of the fuel or core component through use of qualified inspectors (Qualified by Reactor Engineering Annually) working to station procedures that as a minimum accomplish all requirements of the receipt procedure. |
| Wolf Creek | Official inspection is performed by QC. Rx Engineer looks also as a second check but only takes care of the paper work requirements and directs the receipt process. |
| Oconee | RxE who are qualified as Nuclear Fuel Inspectors |

Question 4: What specific role(s) does Rx Engineer perform?

| Clinton | Reactor Engineering supports the project manager (Reactor Services) by preparing the move sheets, overseeing the logistics (contacting the vendor if needed), and when necessary supporting Mechanical Inspection with inspections. The role of the Reactor Engineer is merely now a support organization to ensure that the process is working. |
| --- | --- |
| Cooper | The RE at Cooper plans and coordinates, and implements fuel receipt and inspection. Although the SRO is the ultimate authority on the floor, the RE acts as a second supervisor and signs as SNM checker on the move sheets. Also, all of our REs are qualified as Level II fuel inspectors. |
| River Bend | Limited oversight and SNM accounting. |
| Grand Gulf | SNM tracking; arrangement of shipments; 741's |
| Waterford | Reactor Engineering performs inspections, determines the location for new fuel, and provides oversight of the new fuel receipt process. |
| Fitzpatrick | Provide fuel move sheets to dry storage or pool and process SNM paperwork. |
| Byron | Reactor Engineering provides the interface between the corporate Nuclear Fuels Fuel Reliability Engineer, the fuel vendor and the organization (Reactor Services) directly responsible for inspecting fuel and associated core components and assures all fuel bundles and associated core components are inspected and the inspection results are reconciled to the design documents and all identified discrepancies are resolved. |
| Wolf Creek | Coordinates over-all receipt process and provides fuel move paperwork for storage in dry storage or spent fuel pool. Provides all necessary SNM accountability. Coordinates between Fuel Fabricator, Maintenance, and Operations personnel for unloading the containers and receiving them on site. |
| Oconee | Receipt inspector checking the assembly and accepting it for the site prior to going into the pool (no new fuel vault for ONS.) |

Question 5: Who unloads shipping containers?

| Clinton | Shipping containers are unloaded by the Mechanical Maintenance Department. |
| --- | --- |
| Cooper | Maintenance unloads the containers under the supervision of the RE and an SRO. |
| River Bend | Maintenance and RP. |
| Grand Gulf | Maintenance |
| Waterford | Mechanical maintenance under the direction of the refueling supervisor or reactor engineering. |
| Fitzpatrick | Maintenance |
| Byron | Reactor Services |
| Wolf Creek | Operations and Maintenance provides the man power for the actual activity. |
| Oconee | Maintenance (Fuel Handling Team) |

Question 6: Does your plant utilize the same process for fuel receipt at the site as other procured material?

| Clinton | No, Fuel Receipt Activities are actually governed under their own procedures and processes that are connected but is entirely separate from the processes of procuring materials. |
| --- | --- |
| Cooper | No. Fuels and Reactor Engineering procedures specifically govern receipt/inspection of new fuel, control blades, and fuel channels. |
| River Bend | Unknown. |
| Grand Gulf | No. Basically, the vendor's process is accepted for QA purposes. |
| Waterford | No. Other procured materials go through the warehouse. |
| Fitzpatrick | No Response |
| Byron | Fuel receipt does not utilize the same process. All other procured materials goes through the Stores Department and utilizes QC for safety related parts and equipment. The fuel is taken directly to the Fuel Handling Building to be unloaded and begin the receipt process. |
| Wolf Creek | Pretty well the same with some exceptions because the containers cannot be opened until they are in the fuel building, so security and HP requirements are a little bit different. |
| Oconee | Essentially the same Supply Chain processes / procedures but with a specific inspection procedure for fuel receipt. Other materials are receipt inspected and accepted at the warehouse. |

Question 7: Does your plant require the fuel receipt personnel to be ANSI/ASME N45.2.6 qualified, and at what level? If not required to be qualified to the N45.2.6, why not?

| Clinton | No. There is no formal requirement for our inspectors to be qualified to ANSI N45.2.6. |
| --- | --- |
| Cooper | Yes, our QA department requires us to meet those requirements. Everyone qualified to inspect fuel is a minimum Level II QC inspector. |
| River Bend | Unknown. Please check our procedure or call Kahnne Bankston (QA) at RBS (225-381-4745). |
| Grand Gulf | NO. This is not a QA inspection and we have made no commitment for Level III qualification. We used to do it, but decided it was not required. |
| Waterford | Not currently as it’s not required under current QA program manuals. Entergy is examining the applicability of ANSI/ASME N45.2.6 to our fuel receipt inspections and whether it would benefit the receipt process or if it is required. |
| Fitzpatrick | They have to be task qualified which I believe meets the ANSI requirements. |
| Byron | Not currently. Exelon is examining the applicability of ANSI/ASME N45.2.6 to our fuel receipt inspections and whether it would benefit the receipt process or if it is required. |
| Wolf Creek | QC and RX are qualified. I do not know if there is a special requirement for Maintenance personnel. They are not specially qualified above their normal work except they go through the necessary pre-job briefs before they perform the activity. We keep the same Operations and Maintenance personnel for the entire new fuel receipt activity. |
| Oconee | Unknown, may be embedded in the base documents that they meet ANSI N45.2.6. |